DETAILED ASSEMBLY WORK INSTRUCTIONS (SOP)

Document ID: SOP-ASSY-AETH100-REV-A

Project Title: Aether100 Pro Edge AI Workstation Hardware Platform

Revision: Rev A

Classification: Standard Operating Procedure (Manufacturing Line)

REVISION HISTORY

| Rev | Description of Change | Prepared By | Approved By | Release Date |
| --- | --- | --- | --- | --- |
| A | BaselineAssemblyWorkInstructionsRelease | LeadManufacturingEng | ProductionSupervisor | 2026-07-31 |

1. SAFETY WARNINGS & ESD PRECAUTIONS

● MANDATORY ESD SAFETY: OPERATOR MUST WEAR A CALIBRATED GROUNDING WRIST

STRAP ( RESISTOR) CONNECTED TO A VERIFIED EARTH GROUND POINT PER

ANSI/ESD S20.20.

● PPE MANDATE: SAFETY GLASSES (ANSI Z87.1) AND CUT-RESISTANT GLOVES MUST BE

WORN DURING ALL ASSEMBLY OPERATIONS.

● TORQUE WARNING: CALIBRATED TORQUE DRIVERS MUST BE USED FOR ALL FASTENERS.

DO NOT EXCEED SPECIFIED TORQUE LIMITS TO PREVENT THREAD STRIPPING IN

ALUMINUM.

2. REQUIRED TOOLING & MATERIALS

● Torx Screwdrivers: Calibrated Micro-Torque Drivers (Torx T8, T10)

● Chemicals & Adhesives: Loctite 243 (Medium Strength Threadlocker), Isopropyl Alcohol

(99.9% IPA)

● Thermal Management: Laird Tflex HD9000 Pre-cut Thermal Interface Pads

● Standard Time Target ( ): 18.5 minutes per complete workstation assembly.

3. STEP-BY-STEP ASSEMBLY PROCEDURE

| Step | Op Time | Operation Descriptio n | Fastener / Adhesive | Torque Specificati on | Critical Verificatio n |
| --- | --- | --- | --- | --- | --- |
| 010 | 2.0 min | Clean top aluminum housing interior with 99.9% IPA. | Wipes / IPA | N/A | Zero lint / oil residue |
| 020 | 3.0 min | Apply Laird HD9000 thermal pad to vapor chamber base. | LairdHD9000Pad | HandPressure | Zero air bubbles |
| 030 | 3.5 min | Align PCBA main logic board onto chassis standoffs. | N/A | N/A | Align pins engaged |
| 040 | 4.0 min | Secure PCBA using 8x M2.5 Torx screws in star pattern. | M2.5 TorxScrews | Star torque pattern |  |
| 050 | 2.5 min | Install dual M.2 PCIe Gen5 NVMe SSDs into slots M1/M2. | M2.0 NVMeScrews | insertion angle |  |
| 060 | 2.0 min | Connect 140mm silent fan | Cable Latch | Hand Latch | Positive click engage |

| header to PWM connector J4. |
| --- |
| 070 | 1.5 min | Secure bottom base plate using 8x M3 Torx screws. | M3 Torx +Loc 243 | Flush seam alignment |

4. DETAILED OPERATION VISUAL & TORQUE

EXECUTION

Operation 040: PCBA Fastener Star Sequence

To prevent PCB warping and ensure uniform thermal pad compression, tighten the 8x M2.5 screws using a cross-pattern star sequence:

[1] [7]

\ / +-------+ | ACE-1 | | BGA | +-------+

| / [5] | \ | [3] |
| --- | --- | --- |

1. Set torque driver to ( ).

2. Execute initial pass to across positions 1 through 8.

3. Execute final pass to across positions 1 through 8.